GENERAL RISK ASSESSMENT TEMPLATE

| Work area / operation | CB11.10.403 |  | Assessor’s name | JACK HEFFERNAN, JAMES WITCOMB, WILL RICHARDSON |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Other persons consulted | Zain Khan |  |  |  | Date of safety assessment |  |  |
| Subject Coordinator’s Name | Gavin Paul | Lab Supervisor’s Name |  |  |  | Felipe Ulloa Gutierrez |  |

| ACTIVITY - Describe hazardous activities related to the work area or operation. | ASSOCIATED HAZARDS | INHERENT RISK - Harm that could occur from these hazards if controls fail or are not in place. | EXISTING CONTROL MEASURES | PROPOSED CONTROL MEASURES - Proposed action to minimise risk to an acceptable level. | TARGET DATE - To implement proposed controls | RESIDUAL RISK LEVEL (H,M,L) |
| --- | --- | --- | --- | --- | --- | --- |
| Electrical cables mishandled | Tripping, electrical shock, cable damage | Trip and falling, electric shock. | Cables routed in safe ways, insulated wiring. | Cable management, Visual inspection, no cables across walkways | Before lab sessions (ASAP) | L |
| Leaving the robot arm unattended | Uncontrolled and unexpected movement, collision with bystanding. | Unexpected arm movement hitting people, collision with equipment. | E stop, supervision from tutors. | Don’t leave the robot unattended, power down and utilise e stop before leaving. | Before lab sessions (ASAP) | L |
| Using inappropriate load on the robot | Mechanical failure, arm overload, dropping payload. | Mechanical failure or joint damage. | Manager payload specifications. | Check payload ratings before using robot. | Before lab sessions (ASAP) | L |
| Robot arm moves incorrectly | Collision with environment, unexpected trajectories | Collision with operator or nearby equipment. | Simulation settings, before the robot is run, E stop. | Simulate robot before using physically, low speed run before normal run speed. | Before lab sessions (ASAP) | L |
| Incorrect manual handling | Injury to robot from lifting robot in unintended ways | Injury to robot from moving the robot incorrectly. | Lab equipment being in a fixed position usually. | Manual handling training guidance from tutors, use trolley or wheels when moving robot. | Before lab sessions (ASAP) | L |
| Operating robot when tired or distracted | Human error from incorrect commands | Operator or environment colliding with robot. | Scheduled supervision from markers. | Don’t use the robot when tired, use a buddy when using the robot. | Before lab sessions (ASAP) | L |
| Programming while robot is active | Accidental motion leading to unintended movements from robot. | Operator being hit and moving in other unintended ways within workplace. | Reduce speed mode, Supervision, E stop. | Stay outside of robot range whilst coding, upload code whilst machine is powered down or in emergency stop mode. | Before lab sessions (ASAP) | L |
| Pick-and-place of bar props | Gripper drops or flings item mid-move, pinch points at gripper fingers | Operator struck by object, pinched fingers, item broken | E stop, reduced speed mode | Plastic props only (no glass), stay under payload, test grip at low speed first, keep hands clear of gripper while powered | Before lab sessions (ASAP) | L |
| E-stop / PLC wiring | Incorrect wiring means E stop doesn't cut robot motion | Robot keeps moving during an emergency, operator hit | Supervision from tutors | Test E stop and reset/recovery before every run, supervisor checks wiring before powering on | Before lab sessions (ASAP) | L |
| Running sim code on the real robot | Real base/table positions differ from simulation, unexpected trajectories | Arm collides with table, props or operator | Simulation, E stop | Measure real setup and match it in sim, jog to first pose manually at low speed, dry run with no props first | Before lab sessions (ASAP) | L |
| Working near the moving arm (entanglement) | Loose clothing, long hair or lanyards caught by arm or gripper | Entanglement, being pulled into robot | Lab safety brief | Tie back long hair, remove lanyards and loose clothing, no gloves near moving arm, stay outside robot range while running | Before lab sessions (ASAP) | L |
| Liquids near equipment | Spill onto controller/PLC, wet floor | Electric shock, slipping | Spill kit in lab | Electrics kept away from liquids, paper towels on hand, clean spills immediately | Before lab sessions (ASAP) | L |
| Arduino (If Selected for Emergency Stop) | Short Circuit, Incorrect wiring | Electric Shock, Fire Hazard, Damage to expensive equipment | 20V Max working limit. 5v Arduino max without external power | Inspect Wiring before running code and ensure that it is correct. Ensure no Liquids that could cause short circuit. No 5v to Ground | Before lab sessions (ASAP) | L |
| Controller Battery (If Selected for emergency stop) | Battery Risk, | Electric Shock, Fire Hazard, Damage to expensive equipment | Safety standards by manufacturer | opt to use AA Batteries instead of lithium rechargeables which are less dangerous | Before lab sessions (ASAP) | L |

| Approval of assessment | I am satisfied that the residual risk with existing controls is acceptable X Yes ☐No OR I am satisfied that that the proposed controls will reduce risk to an acceptable level. X Yes ☐No | Signature | JACK HEFFERNAN, JAMES WITCOMB, WILL RICHARDSON | Date | 21/08/2026 |
| --- | --- | --- | --- | --- | --- |

Guidance notes for documenting General Risk Assessments

ACTIVITY

Briefly describe this hazardous work activity - E.g. Operating, Handling, Using … (Include names) of hazardous equipment, substances or materials used, and any quantities and concentrations of substance(s) or reaction products.

ASSOCIATED HAZARDS

Plant & Equipment – noise, vibration, moving parts (crushing, friction, stab, cut, shear), pressure vessels, lifts/hoists/cranes, sharps

Manual Handling – repetitive movements, lifting awkwardly, lifting heavy objects

Work Environment – moving objects, extremes in temperature, isolation, work at height, allergies to animal bedding, dander and fluids, risk of fire/explosion, slippery surfaces/trip hazards

People – potentially violent or volatile clients/interviewees

Communicable Diseases – exposure to bodily fluids/infectious materials, animal bites and scratches,

Environmental – emissions to atmosphere, discharge to soil and water bodies (including stormwater run-off), nuisance noise & odour, poor ventilation/air quality

Radiation (non-ionizing) – including lasers, microwaves or UV light

Electrical – plug-in equipment used in ‘hostile’ work environment, exposed conductors, high voltage equipment

Pathogens – dealings with pathogenic microorganisms such as bacteria, parasites, fungi or viruses

GMOs – dealings with genetically modified organisms

Cytotoxins – carcinogens, mutagens or teratogens

Radiation (ionizing) – Ionizing radiation source such as radioactive substance or radionuclide, or irradiating apparatus

Chemical – hazardous substances, dangerous goods, fumes, dust, compressed gas, hazardous waste

INHERENT RISK

Provide details of the harm that could be caused to people or the environment if something goes wrong.

For example: inhalation of fumes, laceration, injury to back, infection, burns to skin or eyes.

Think about what could happen if controls fail or are not in place.

CONTROL MEASURES

Note the existing and proposed actions to reduce risk to an acceptable level. Apply the “Hierarchy of Controls”, listed below, when deciding the best control measure to apply. Control types closer the top of the list are preferable.

1. Eliminate the hazard. For example: use a different less dangerous piece of equipment, fix faulty machinery, use safer materials or chemicals

2. Isolate the hazard from the people. Separate people from the danger.  For example: use shielding, use lifting equipment or trolleys, remove dust or fumes with exhaust system, lock-out machinery.

3. Change the way the job is done. For example: change work practices, provide training, information and signs, develop work procedures.

4. Use personal protective equipment (PPE), noting specific PPE is required for each job. For example: respirator, hearing protection, gloves. Training and information is required for the use of PPE.

RESIDUAL RISK LEVEL (H, M, L)

Estimate risk taking into account the way the activity is run and control measures put in place. The level of risk can be determined by combining consequence and likelihood using the risk matrix from below. Residual risk should be reduced to a level acceptable by management.

CONSEQUENCE OF HARM - This is how bad it will be if something does go wrong e.g. the number of people that could be harmed, the severity of injury.

LIKELIHOOD OF HARM - Chance of harm occurring is affected by the duration of the activity and its frequency; the number of people doing the activity and the level of exposure to the hazard.